Why design reactors?

- Control reaction conversions

- Cannot arbitrarily control reaction mechanisms or rates

- Issues of reactor size, mass transport, thermal control, etc. vs. economics

Ideal Batch Reactor

- Constant volume

- Uniform composition, varies with time

- Isothermal

Steady State Flow Reactors

Mixed Flow Reactor

- Uniform composition in position

- Composition does not vary with time

- Outlet concentration = reactor concentration

- Constant volume

Plug Flow Reactor

- Composition varies with axial position

- Composition does not vary with time

- Constant volume

- No axial mixing

- Homogeneous radial composition

Mass Balance

Accumulation = In - Out + Generation - Consumption

0 = In - Out - Consumption

For batch reactor: flows = 0, for mixed flow/plug flow, acceleration = 0

Ideal Batch Reactor

Accumulation = In - Out + Generation - Consumption

Accumulation = - Consumption = -NA0 * dXA/dt = -rAV

t = NA0 ∫0XA 1/-rA dXA

Constant Volume

t = CA0 ∫0XA 1/-rA dXA = -∫CA0CA 1/-rA dCA = reaction time

Space Time and Space Velocity: performance measure for MFR, PFR

Space time: 𝜏 = 1/s, time to process one reactor volume, units: time

Space velocity: s = 1/𝜏, reactor volumes in unit time, units: time-1

Holding Time and Space Time

𝜏 = reactor volume/volumetric flow rate = CA0 * V / molar flow rate

Ideal Steady State MFR

- Composition uniform

- Exit conditions = reactor conditions

In - Out - Reacted = 0

CA0v0 - CAv0 + VrA = 0

-VrA = CA0v0 - CAv0

V/v0 = CA - CA0/rA

𝜏 = CA-CA0/rA

𝜏 = V/v0 = CA0V/FA0 = CA0XA/-rA = CA0 - CA/-rA

First Order MFR

k𝜏 = XA/(1-XA) = CA0 - CA/CA

CA = CA0/(1+k𝜏)

XA = k𝜏/(1+k𝜏)

Second Order MFR

k𝜏 = CA0 - CA / CA2

CA = (-1 + [1 + 4CA0k𝜏]1/2)/2k𝜏

-rA = kCA2

-1/rA = 1/kCA2

𝜏 = CA0XA/-rA = CA0XA/kCA2

CA2 = CA0XA/k𝜏

Practice Problem

𝜏  = 1 min

CA0 = 0.1 mol/L

CB0 = 0.01 mol/L

CAf = 0.02 mol/L

CBf = 0.03 mol/L

CCf = 0.04 mol/L

V = 1 L

Disappearance: -rA = CA0-CA/𝜏 = 0.10 - 0.02 / 1 = 0.08 mol/L min

Formation: -rB = 0.02 mol/L min; -rC = -0.04 mol/L min